Target Components: 1x Stepper, 4x Servos, 1x IR Sensor, 1x USB Camera

- Power Distribution and Regulation The PCB must handle two primary voltage rails. Total peak current is estimated at 3.5A.

12V Rail (Primary Input):

- Stepper Motor: 0.35A per phase

5V Rail (Regulated):

- 4x Hobby Servos: 500mA peak each (2A total)

- USB Camera: 260mA max

- IR Sensor: 20mA max

Engineer Note: Use a high-efficiency buck regulator for the 5V rail (min 3A rating) and include a 470uF electrolytic decoupling capacitor near the servo headers to handle transient spikes.

- Component Interface Details

Hobby Servo (Adafruit 169): 4 units. Use male shrouded headers with 2.54mm (0.1 inch) pitch. Wiring: 1: Signal (Orange), 2: 5V (Red), 3: GND (Brown).

Stepper Motor (XY42STH34): 1 unit. Use JST-VH or Molex KK headers with 3.96mm pitch. Wiring: 1: Red (A), 2: Yellow (C), 3: Gray (B), 4: Green (D).

IR Break Beam (Adafruit 2168): 1 unit. Use male headers with 2.54mm (0.1 inch) pitch. Wiring: 1: 5V (Red), 2: GND (Black), 3: Signal (White).

USB Camera (DFRobot FIT0729): 1 unit. Use 4P-2.0mm header with 2.0mm pitch. Interfaces with USB Data plus/minus and 5V/GND.

- Mandatory Protection and Logic IR Sensor Logic: The receiver is Open Collector. The PCB must include a 10K Pull-up Resistor between the Signal (White) and 5V (Red) lines.

Stepper Control: Include footprints for an A4988 or DRV8825 driver module to interface with the 4-lead motor.